# ROCO318 Coursework Report – Bipedal Walking Robot

# Table of Contents

- Introduction………………………………………………………………... 1

- Understanding and Controlling RC Servomotors……………………….… 1

- Mechanical Design of the Bipedal Robot…………………………………. 2

- Fabrication and Assembly…………………………………………………. 4

- Gait Pattern Development…………………………………………………. 4

- Feedback Control and Sensor Integration…………………………….…… 4

- Running Test Results………………………………………………...…….. 5

- Conclusion…………………………………………………………………. 5

- Appendices ………………………………………………………………... 6

# 1. Introduction

The goal of this project was to build a bipedal walking robot, and through this to develop a deeper understanding of servo control, CAD design, fabrication, gait control and feedback. This is very important as the field of bipedal / humanoid robotics is growing rapidly at the moment so experience in it is valuable to our professional careers.

# 2. Understanding and Controlling RC Servo Motors

## 2.1 RC Servo Characteristics

# RC servo motors are extremely useful for building robots as they move quickly and precisely to desired angles due to their inbuilt encoders. Thanks to the numerous pre-existing libraries they are also very simple to set up and control in a basic fashion.  They make implementing inverse kinematics trivial as you can simply set them to the angle your inverse kinematics equations tells you they should be, and your end effector should be in the right place.

## 2.2 Microcontroller Setup

# In our robot, we used an ESP32 to control a PCA9685 Pulse Width Modulation (PWM) break out board that controlled our eight servo motors. The PWM breakout board sends a PWM signal to the servo motor and the width of the pulse determines the angle that it will go to. The ESP32 communicates with the PCA9685 through the I2C protocol, this means that we only need to use 4 pins on the ESP32 to control up to 16 servo motors. We did not need to do any of the programming of the I2C ourselves as the libraries from Adafruit and Arduino handled it all for us, and to set the servo motors using degrees instead of pulse lengths, we just had to calculate the appropriate scaling factor.

## 2.3 Servo Control Code

# If the servo motors are set to move in a big jump between different angles the movement can be very jerky and the position of the end effector becomes unpredictable. Both are unsuitable for balancing and controlling a servo motor. We initially combatted this by splitting each movement down into smaller steps, however this approach can either be very slow, or have lots of jerky steps as it stops and starts. We them decided to implement a proportional controller that calculated the error between the current servo position and the target location and the calculated a step to move the servo towards the target, with step size proportional to the error. This allowed the servos to move quickly across big distances, but to also slow down as they reached the target position to avoid an abrupt halt that causes wobble in the robot. This greatly improved the stability of the robot, while not making it slow. This code can be found in the “Servo_Code” folder in the “servo_code.ino” file.

## 2.4 Experimental Demonstration

In Appendix A you can see how we have precise control of the robot as the body stays level while the knees bend to allow it to go up and down. You can also see the difference between code with and without smoothing. This shows that our code is accurate and effective.

# 3. Mechanical Design of the Bipedal Robot

## 3.1 Design Requirements

Each group was given eight servos and had to design a bipedal robot to walk one metre in the fastest time possible. The only limit was a maximum limb length of 10 centimetres. We intended to make a design with high static stability and a high range of movement, so we designed it to be similar to a chicken.

## 3.2 CAD Design Process

For our CAD we used Autodesk Fusion as we have both used it previously. Our final version does not differ greatly from our initial design, however there are a few key differences. When first discussing the design, we decided to try and keep all servos as high as possible so that we had more mass to move around for balance. Because of this we had one servo in each knee and then we put two servos into each hip. We also planned to use one servo to create a tail to aid in balancing in one axis and another to move a head to balance in another axis. We got as far as designing the tail before realising that the servos would be better used in the hips to give them three degrees of freedom each. Once we got rid of the head and tail we redesigned the hip plate to accommodate them and modelled mounting points for the electronics above them.

We had a few iterations for the legs, we first wanted to use a four-bar linkage to keep the feet flat without the use of an additional servo, but this proved too complex and too fragile so we changed just a standard leg directly controlled by a servo. Initially we also tried to create an actuated ankle using rubber bands instead of a servo (Fig. 1) however upon testing we discovered that the bands tended to try and pull the foot to one of the limits instead of keeping it stable in the centre. Instead of the actuated foot we created a “dumb” foot that simply bolted onto the end of the lower leg piece (Fig. 2). We first tried to create a foot that mimicked a chicken’s foot however this proved too unstable, so we added extra spokes to the back of the foot in order to widen the support polygon while the robot is on one foot.

| Fig. 1 – Rubber Band Actuated Ankle | Fig. 2 – “Dumb” Foot |
| --- | --- |

## 3.3 Final CAD Model

Our final model is very lightweight which helped in the rapid prototyping phase as parts could be reprinted very quickly. Another good aspect of the design is that the left and right legs are completely identical up until the hips, which made fabrication and assembly simpler and allowed for spare parts to be readily available. The servos in the hip should allow the robot to lean over one foot to balance while walking to take a step, and the other servos can move the other foot to take the step. Unfortunately, due to the lightweight nature of the design the robot was too unstable to accomplish this as it wobbled too much when trying to stand on one foot and instead fell over. Another solution could be to add some bearings so that the joints could be on both sides of each servo instead of everything just being attached to the servo horn. Potentially, if we made the robot out of metal, it would be more rigid and so would have less problems balancing.

Fig. 3 - Final CAD Model of the Robot

# 4. Fabrication and Assembly

The initial print and the rapid prototyping was done on an Ender 3 Pro, however we had some problems with the strength of the parts and so towards the end of the fabrication process we switched to printing on a Bambu Lab A1S which gave much better results in terms of print quality and strength. All parts were printed in PLA as it is readily available and perfectly suitable for our needs. We could easily export STL files from Fusion then slice and print them very quickly. In order to make the parts stronger we experimented with giving the parts 100% infill which greatly improved strength but also increased print time.

Assembly of the robot was straightforward but time consuming as there were lots of fiddly screws and bolts, however when it was fully assembled, it was very secure. The robot had to be assembled from the hips down as the hip servo brackets blocked the attachment of the previous servo horn screw once the servo was bolted to the bracket. The Esp32, IMU and PWM boards all had their own cut-outs so we could take them on and off as required. We did have to reassemble once as we initially assembled the parts without setting the servo motors to their initial positions.

# 5. Gait Pattern Development

We did implement forward and inverse kinematics in MATLAB, however we did not end up using them in the control of our robot’s gait. We instead manually programmed and tested the robot’s joint angles to first have it standing in a stable stance and then move through a cycle of set positions. This was hampered by being unable to find a position in which the robot supported itself on one leg so we had to resort to a shuffling motion as we couldn’t move either foot fully clear of the ground. We did try to use our hip servos to move the centre of mass of the robot over the foot that stayed on the ground, however the inherent flexibility of our design made this impossible. In the end our robot could shuffle the whole metre in one minute and 27 seconds as seen in Appendix B.

# 6. Feedback Control and Sensor Integration

The IMU we used was the MPU6050 which include a 3-axis accelerometer as well as a 3-axis gyroscope. This is very useful for a bipedal robot as these can both be used to measure the angle of the robot and they can be combined to give a more accurate measurement.

We used the sensors within a PID controller to stabilise the position of the torso. First, we read the sensors, then used the tanh function on the two relevant accelerometer axes to calculate the angle. We then multiplied the oscilloscope reading by the time between readings to get the angle rather than the angular velocity. We then fed these readings into a complementary filter to combine them. We then took that result and fed it into a PID controller that calculated the error between the measured angle and the desired angle and gave an output of a step that the hip motors should take to keep the robot upright. This code can be found in the “IMU” folder. Unfortunately, we never worked out all of the kinks in the system and so we never had a good example of it working as it would work for a while and then the error value would exponentially increase, and the servos would go to their limits.

# 7. Running Test Results

The running test was slightly disappointing as on our first attempt there were some power issues and the robot stopped at 80 centimetres, however we fixed them and managed to get the robot to travel the full metre. This was at a slow time of over 2 minutes, which is slower than the time we recorded in Appendix B. We think that part of this was caused by the table being slightly uneven which resulted in the robot going off course. This shows that our gait is not very resilient and our results are not very repeatable.

# 8. Conclusion

I am very pleased that in the end our robot was able to complete the task of walking a metre as we put a lot of work into making sure that it would be able to. I am disappointed that we never got the IMU to fully work as I feel that we were very close, perhaps we just needed to tune the P, I, and D values better. Our servo control was very good, and I am happy with how well our P controller worked in smoothing out the movement. In future I would try and make a more stable frame for robot so that it would be able to stand on one foot and so take a proper step. I feel that we underutilised the torque of the motors, I believe that they would easily cope with a heavier design. Moving one of the servos from each side down to the foot would have also aided in balancing which was our biggest challenge throughout the project. I believe it is the biggest challenge facing bipedal robots in general. We are only now seeing it being reliably solved in the newest bipedal robots with them making use of machine learning instead of having a rigid set of steps to follow.

# 9. Appendices

## Appendix A – Video of Robot Squatting

https://youtube.com/shorts/oxxIlt7pca4?feature=share

## Appendix B – Video of Robot Walking 1 Metre

https://youtu.be/SJBYeXuyQ8s